Lab 5 – Challenge Lab with Switch Security Configuration

Total points: 100

Topology

Note: This is a challenge lab that only provides assistance with switch security configuration commands. For help with other configuration tasks refer to your previous labs.

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0/1.10 | 192.168.10.1 | 255.255.255.0 | N/A |
|  | G0/0/1.20 | 192.168.20.1 | 255.255.255.0 | N/A |
|  | G0/0/1.99 | 192.168.99.1 | 255.255.255.0 | N/A |
| S1 | VLAN 99 | 192.168.99.11 | 255.255.255.0 | 192.168.99.1 |
| S2 | VLAN 99 | 192.168.99.12 | 255.255.255.0 | 192.168.99.1 |
| PC-A | NIC | DHCPv4 |  |  |
| PC-B | NIC | DHCPv4 |  |  |

- Switch Port Assignment Specifications

| Ports | Assignment |
| --- | --- |
| S1 F0/1, F0/5 | 802.1Q Trunk |
| S2 F0/1 | 802.1Q Trunk |
| S1 F0/6 | VLAN 10 |
| S2 F0/18 | VLAN 20 |
| S1 and S2 unused switch ports | VLAN 1000 |

- Objectives

Part 1: Build the Network and Configure Basic Device Settings

Part 2: Configure Switches with VLANs and Trunking

Part 3: Configure Trunk-Based Inter-VLAN Routing

Part 4: Configure DHCPv4

Part 5: Configure Switch Security

- Secure Unused Switch Ports

- Implement Port Security Features

- Implement PortFast and BPDU guard

Part 6: Verify end-to-end connectivity

Build the Network and Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings on the switches and the router.

Cable the network as shown in the topology.

Configure basic settings for each switch.

Configure the device name.

Disable DNS lookup.

Assign class as the privileged EXEC mode password.

Assign cisco as the console and vty password and then enable login. Allow the switch to accept inbound Telnet connections.

Encrypt plain text passwords.

Configure a message of the day (MOTD) banner.

Configure basic settings for the router.

    - Configure the device name.

    - Disable DNS lookup.

    - Configure password encryption.

    - Assign class as the privileged EXEC password.

    - Assign cisco as the console and vty passwords and then enable login. Allow the router to accept inbound Telnet connections.

    - Configure a MOTD banner to warn users that unauthorized access is prohibited.

    - You will configure router subinterfaces in Part 3.

- Note: If you need help with basic settings on the switch, refer to the Basic Device Settings file. Use only the commands that you need.

Configure Switches with VLANs and Trunking

In Part 2, you will configure the switches with VLANs and trunking.

Configure VLANs and trunking on S1.

    - Create and name all VLANs as per the VLAN key.

    - Configure the management IP address (SVI) on S1 using the Addressing Table. Add the default gateway information.

Configure trunking on F0/1 and F0/5. Specify that only VLANs 10, 20, and 99 are allowed to cross the trunk. Specify VLAN 99 as native.

Configure F0/6 as an access port and assign it to VLAN 10.

Configure VLANs and trunking on S2.

    - Create and name all VLANs as per the VLAN key.

    - Configure the management IP address (SVI) on S2 using the Addressing Table. Add the default gateway information.

Configure the interface F0/1 as a trunk. Specify that only VLANs 10, 20, and 99 are allowed to cross the trunk. Specify VLAN 99 as native.

Configure F0/18 as an access port and assign it to VLAN 20.

Configure Trunk-Based Inter-VLAN Routing

In Part 3, you will configure R1 to route to multiple VLANs by creating subinterfaces for each VLAN.

- Activate the G0/0/1 interface.

- Configure subinterfaces for VLANs 10, 20 and 99. Specify 99 as native VLAN in the configuration.

- Configure a DHCPv4 Server

  - Configure R1 as a DHCPv4 server.

To automatically assign address information on the network, you will configure R1 as a DHCPv4 server.

Configure a DHCP pool for each of the user VLANs 10 and 20. Use the pool name R1G0 for VLAN 10 and R1G1 for VLAN 20.

Exclude the first 20 addresses in each DHCP pool starting with .1. Make sure that each DHCP address pool includes a network statement to define the range of available addresses, the default gateway, the domain ccna-lab.com, and a DNS server 209.165.200.225. Addresses must be leased for a period of 5 days.

  - Record IP settings for PC-A and PC-B.

Configure PC-A and PC-B for DHCP. From the command prompt, issue the ipconfig command to verify that the PCs have received IP address information from the DHCP server on R1.

Configure Switch Security

Secure Unused Switch Ports

On both switches S1 and S2, assign all unused ports to the ParkingLot VLAN, configure them for static access mode, and administratively deactivate them using the interface range command.

Implement Port Security Features.

The interfaces F0/6 on S1 and F0/18 on S2 are configured as access ports. In this step, you will also configure port security on these two access ports.

    - On S1, enable port security on F0/6. Set the maximum number of secure MAC addresses to 2. Also, enable sticky learning to add dynamically learned MAC addresses to the running configuration.

S1# config t

S1(config)# interface f0/6

S1(config-if)# switchport port-security

S1(config-if)# switchport port-security maximum 2

S1(config-if)# switchport port-security mac-address sticky

S1(config-if)# exit

S1(config)#

    - b.  Verify that the MAC address of PC-A has been automatically added to the running configuration for S1.

- S1# show run

- <output omitted>

- Interface FastEthernet 0/6

- switchport mode access

- switchport port-security

MAC address of PC-A

swicthport port-security maximum 2

- switchport port-security mac-address sticky

- switchport port-security sticky xxxx.xxxx.xxxx

- <output omitted>

Note: If the MAC address of PC-A does not reflect in the running configuration of S1, issue a ping from PC-A to the management IP address of S1. Do not proceed to the next step without this result.

    - c.  Issue the show port-security interface f0/6 and observe the output.

- S1# show port-security interface f0/6

Port Security              : Enabled

Port Status                : Secure-up

Violation Mode             : Shutdown

Aging Time                 : 0 mins

Aging Type                 : Absolute

SecureStatic Address Aging : Disabled

Maximum MAC Addresses      : 2

MAC address of PC-A

Total MAC Addresses        : 1

Configured MAC Addresses   : 0

Sticky MAC Addresses       : 1

Last Source Address:Vlan   : xxxx.xxxx.xxxx:10

Security Violation Count   : 0

    - Similarly, enable port security on S2 F0/18. Set the maximum number of secure MAC addresses to 2. Also, enable sticky learning to add dynamically learned MAC addresses to the running configuration.

    - Verify that the MAC address of PC-B has been added to the running configuration for S2.                          Note: If the MAC address of PC-B does not reflect in the running configuration of S2, issue a ping from PC-B to the management IP address of S2.

    - Verify port security on S2 F0/18 by issuing a show port-security interface command.

Implement PortFast and BPDU guard.

- Configure PortFast and BPDU guard on S1 F0/6.

S1# config t

S1(config)# interface f0/6

S1(config-if)# spanning-tree portfast

S1(config-if)# spanning-tree bpduguard enable

S1(config-if)# exit

S1(config)#

- Similarly, configure Portfast and BPDU guard on S2 F0/18.

- Verify that BPDU guard and PortFast are enabled on the appropriate ports.

S1# show spanning-tree interface f0/6 detail

Port 8 (FastEthernet0/6) of VLAN0010 is designated forwarding

Port path cost 19, Port priority 128, Port Identifier 128.6.

<output omitted for brevity>

Number of transitions to forwarding state: 1

The port is in the portfast mode

Link type is point-to-point by default

Bpdu guard is enabled

BPDU: sent 128, received 0

Verify end-to-end connectivity

Each PC should be able to ping the other PC in the topology. Verify and troubleshoot if necessary.

****************************************************************************************************************************

# After the lab is complete, ask your instructor to verify your work. Issue the following commands and have the output ready for the instructor to check. Points will be deducted for missing or incorrect output/configuration.

# *** DO NOT BEGIN CLEAN UP UNLESS YOUR INSTRUCTOR HAS GRADED THE LAB ***

Output of ipconfig on PC-A and PC-B

Output of ping from PC-A to PC-B and vice versa

Output of show port-security interface f0/6 on S1

Output of show spanning-tree interface f0/6 detail on S1

Output of show vlan brief on S1 and S2

Output of show run on S1 and S2